Wireless Cold Chain Monitor

# Product description

- Monitor critical parameters such as temperature and relative humidity in real time.

- Temperature reporting interval can be configured between minimum of once every second to maximum of once every day.

- Each message from sensor data conveys current temperature and battery voltage data.

- WiSense gateway can support up to 64 simultaneous sensor nodes in a single network

- Built on top of our proven Sub-GHz RF mesh platform and RF to Cellular connectivity.

- Wireless communication between gateway and sensor nodes is bi-directional. This allows node behavior to be reconfigured during installation and at any time after installation

# Product Applications

- Can be used anywhere in the cold chain - from large cold rooms to small ice-boxes in warehouses

- Mobile refrigeration units

# Product Technical Specifications

- Input supply voltage: 3.0 V to 4.2V using

1 x Rechargeable LI-Ion / 2x AAA batteries

- Very low standby mode current consumption of 2 uA allows for long battery life

- Sensing temperature range: -50 deg C to +125 deg C

- Accuracy: ±0.5°C ( -10°C to +85°C)

- Maximum transmit power: +13 dBm

- Wireless Operating Frequency India: 865 to 867 MHz, EU: 868 MHz & USA: 900 MHz

- Wireless range: 1km (line of sight)

- Antenna Type: Half-wave dipole, gain: +3dBi

- Dimension: 103 mm x 116 mm x 33 mm

# Our Other Products

- Ambient Humidity and Pressure Sensor node

- Wireless temperature monitor tag

- Wireless load Cell monitor node

- Solar Li-ion Charger board, Wireless weather station

- PIR sensor node, Door sensor node

- Emergency wall buttons

WSN Mesh network

Reefer with Wireless senor nodes & Gateway

Sensor’s data visualized in the cloud

Wireless Door Sensor Node

# Product description

- Small form factor wireless door sensor node

- Message sent on every status change (open/close) of the door to the central coordinator.

- Each message from sensor data conveys door Status, Battery voltage and MAC address of the device.

- Powered by two-coin cells (3V CR2032) which can last more than three years assuming 1 transmission every 5 Minutes.

- The node has an internal PCB antenna

- Very low power consumption(<2uA),

- LED indicator for door Open/Close status

- Software configurable LED light color and blinking interval

# Product Applications

- Facility management

- Industrial door status monitoring

- Security and monitoring purpose

- Home/building automation

- Smart buildings

# Product Technical Specifications

- Input supply voltage: 1.8 to 3.0 V DC

- Current consumption: <2 microamps (Sleep mode)

- Response time: < 1 Second

- Maximum transmit power: +10 dBm

- Wireless range: 70-80 metres

- Antenna Type: PCB

- Operating Temperature: -25° to +100° Celsius

- Data rate: 1.2kbps - 100kbps

- Enclosure Material: ABS

- Certifications: FCC, ETSI, WPC/GOI compliant radio for India

WSN Mesh network

Snapshot of the sensor’s data visualized in the cloud

Wireless Humidity Sensor Module

# Product description

- Wireless humidity sensor uses CC2D33S Amphenol Advanced Sensors measures range 0 to 100 % with

+/- 3% accuracy

- Temperature sensor uses NXFT15XH103 NTC thermistor (Murata) measures range -40 deg C to

+125 deg C

- Each message from sensor data conveys current humidity, temperature and battery voltage data.

- Reporting interval 1 second or more (configurable in real time via network coordinator)

- Powered by 2XAAA Batteries (Inside or outside enclosure)

- Very low standby mode current consumption of 2 uA allows for long battery life

- Sensor operate in the 865-867 MHz license free band in India.

# Product Applications

- Indoor ambient humidity and temperature measurement

- Cold storage, ware houses

- Environmental monitoring

- For HVAC Systems (Heating Ventilation & Airconditioning)

# Product Technical Specifications

- Input supply voltage: 1.5 to 3.0 V DC

- Current consumption: <2 microamps (Sleep mode), 2 micro amps (Active mode)

- Sensing humidity: Range: 0 to 100 %, Accuracy: +/- 3%, No calibration required

- Sensing temperature range: -40 deg C to +125 deg C

- Response time: < 1 Second

- Maximum transmit power: +13 dBm

- Each node has a unique IEEE assigned 64 bit address

- Wireless range: 1.0 k.m (line of sight)

- Antenna Type: Half-wave dipole, Gain: +3 dBi

- Enclosure Material: ABS

- Dimensions: 120 mm x 80 mm x 50 mm

WSN Mesh network

Snapshot of the sensor’s data visualized in the cloud

Wireless Load Sensor Node

# Product description

- Wireless load cell sensor measures load/ weight of materials

- Reports measured load value periodically with configurable interval - Minimum (1 sec) / Maximum (1 day)

- Reports measured load only when it changes by a configurable percentage value with respect to the prior value reported

- Each message from sensor data conveys load values and battery voltage data.

- Powered by 2 x AAA or 2 x AA Batteries or one 1 x Rechargeable LI-Ion (3.7V to 4.2V) battery

- Very low standby mode current consumption of 2 uA allows for long battery life

- Sensor operate in the 865-867 MHz license free band in India.

# Product Applications

- LPG cylinder gas weight (level)

- Industrial weighing gauges

- Fitness equipment

# Product Technical Specifications

- Input supply voltage: 3.0 to 5.0 V DC

- Current consumption: <2 microamps (Sleep mode), 2 micro amps (Active mode)

- Sensing load range: 0 to 1000 Kg

- Accuracy: ± 0.3

- Sensor type: Single point pending beam load cell

- Maximum transmit power: +13 dBm

- Each node has a unique IEEE assigned 64-bit address

- Antenna Type: PCB/ External, Gain: +3 dBi

- FCC/ETSI Certification available

- Portable size

WSN Mesh network

Snapshot of the sensor’s data visualized in the cloud

# Our Other Products

- Cold Chain Monitor node

- Wireless PT100 temperature monitor node

- Solar Li-ion Charger board

- Wireless weather station

- PIR sensor node

- Door sensor node

- Emergency wall buttons

2

Low Profile Wireless Temperature Tag

Product description

- Low profile wireless temperature sensor node WTN2P1 measures -20 deg C to +50 deg C.

- Temperature reporting interval can be configured between minimum of once every second to maximum of once every day.

- Each message from sensor data conveys temperature and battery voltage data.

- Up to 64 such sensor nodes can communicate through one WiSense coordinator node (WSN1120CL)

- Powered by 3V CR2032 coin battery, which can last up to one year assuming 1 transmission every 5 Minutes.

- The tags operate in the 865-867 MHz license free band in India.

- Supports WiSense mesh

# Product Applications

Suitable for sensing indoor temperatures such as

- Pharmaceutical temperature monitoring

- Cold storages

- Environmental monitoring,

# Product Technical Specifications

- Input supply voltage: 1.8 to 3.0 V DC

- Current consumption: <2 microamps (Sleep mode), 2 micro amps (Active mode)

- Sensing temperature range: -20 deg C to +50 deg C

- Accuracy: ±0.5°C

- Resolution :0.0625 deg C

- Response time: < 1 Second

- Maximum transmit power: +13 dBm

- Wireless range: 100 m

- Antenna Type: PCB

- Dimension: 37mm x 29mm x 8mm

WSN Mesh network

Snapshot of the sensor’s data visualized in the cloud

Wireless PIR Motion Sensor node

# Product description

- The Wireless PIR (Passive Infra-red) Motion Sensor detects motion and movement using infrared radiations

- Reports to the central coordinator when a motion is detected

- Each message from sensor data conveys motion detected/not detected status and battery voltage data.

- Powered by 2 x AAA or 2 x AA Batteries or one 1 x Rechargeable LI-Ion (3.7V to 4.2V) battery

- Very low standby mode current consumption of 2 uA allows for long battery life

- Sensor operate in the 865-867 MHz license free band in India.

- Detection area Horizontal 94°, Vertical 82° & Detection distance is 5m

# Product Applications

- Security Systems

- Alarm System

- Motion activated light switch control

# Product Technical Specifications

- Input supply voltage: 3.0 to 5.0 V DC

- Current consumption: <2 microamps

- Operating frequency: 865-867 MHz

- Sensor Detection range: 5 metres

- Sensor detecting Angle: Horizontal 94°, Vertical 82°

- Maximum transmit power: +13 dBm

- Each node has a unique IEEE assigned 64-bit address

- Wireless range: Around 1.0 k.m (line of sight)

- Antenna Type: Half-wave dipole, Gain: +3 dBi

- FCC/ETSI Certified

- Dimensions: 120 mm x 80 mm x 50 mm

WSN Mesh network

Snapshot of the sensor’s data visualized in the cloud

PT100 Wireless Temperature Sensor

# Product description

- PT 100 Wireless temperature Sensors measure the range -200 deg C to +550 deg C with ± 0.5 deg C accuracy

- Reports measured temperature periodically with configurable interval - Minimum (1 sec) / Maximum (1 day)

- Reports measured temperature only when it changes by a configurable percentage value with respect to the prior value reported

- Each message from sensor data conveys temperature and battery voltage data.

- Powered by 2 x AAA or 2 x AA Batteries or one 1 x Rechargeable LI-Ion (3.7V to 4.2V) battery

- Very low standby mode current consumption of 2 uA allows for long battery life

- Sensor operate in the 865-867 MHz license free band in India.

# Product Applications

- Industrial high temperature applications – furnaces, boilers

- Cold storage, ware houses

- Environmental monitoring

# Product Technical Specifications

- Input supply voltage: 3.0 to 5.0 V DC

- Current consumption: <2 microamps (Sleep mode), 2 micro amps (Active mode)

- Sensing temperature range: -220 deg C to +550 deg C

- Accuracy: ±0.5°C (0°C to 85°C)

- Sensor support any 3 wire PT-100/PT-1000 sensor

- Maximum transmit power: +13 dBm

- Each node has a unique IEEE assigned 64-bit address

- Wireless range: Around 1.0 k.m (line of sight)

- Antenna Type: Half-wave dipole, Gain: +3 dBi

- FCC/ETSI Certified

- Dimensions: 120 mm x 80 mm x 50 mm

WSN Mesh network

Snapshot of the sensor’s data visualized in the cloud

# Our Other Products

- Cold Chain Monitor node

- Wireless load Cell monitor node

- Solar Li-ion Charger board

- Wireless weather station

- PIR sensor node

- Door sensor node

- Emergency wall buttons

Wireless Solar Powered Sensor Node

# Product description

- Li-Ion solar charger board for wireless nodes

- Texas Instruments based lithium-ion charger IC facilitates auto switchover to battery when external supply is lost

- Up to 2A switch mode charging operation

- Charging current and voltage monitoring through the onboard I2C interface

- Each message from sensor data conveys Solar panel output voltage & current, battery voltage & current data.

- Charger IC can power the load and charge the battery simultaneously.

- Then node includes a thermistor used to monitor the temperature of Li-Ion battery

- I2C, UART, SPI and Analog interfaces for sensor integration

# Product Applications

- Suitable for applications such as running standalone router/sensor node from solar mains with Li-Ion battery backup.

- Powering all kinds of outdoor electronics like wireless sensor

nodes and associated sensors/actuators etc.

# Product Technical Specifications

- Max input (solar panel) voltage: 10.5 V (Vmp)

- Max charge current: 2 A

- Max battery discharge current: 4 A

- Li-Ion battery charged in 3 phases (trickle charge, pre- charge, constant current and constant voltage)

- Maximum transmit power: +14 dBm

- Multiple output voltages (On separate headers/connectors)

o  ~3.3V (Max 1 A), 4.9V (Max 50 mA)

  - Li-Ion battery output (Max 4A)

- Voltage output at peak power point (Vmp): 8.5V -8.8V

- Each node has a unique IEEE assigned 64-bit address

- Antenna Type: Half-wave dipole, Gain: +3 dBi

WSN Mesh network

Snapshot of Battery voltage data

Wireless Temperature Sensor Tag

# Product description

- Wireless temperature sensor Tag uses Industrial grade sensor with a range -25 deg C to +60 deg C.

- Temperature reporting interval can be configured between minimum of once every second to maximum of once every day.

- Each message from sensor data conveys current temperature and battery voltage data.

- Up to 64 such sensor nodes can communicate through one WiSense coordinator node (WSN1120CL)

- Powered by two coin cells (3V CR2032) which can last more than three years assuming 1 transmission every 5 Minutes.

- The tags have an internal PCB antenna but also have a U.Fl connector for an external whip antenna.

- The tags operate in the 865-867 MHz license free band in India.

# Product Applications

Suitable for sensing indoor temperatures such as

- Ware houses

- Cold chains

- Environmental monitoring,

- Structures and Machines

# Product Technical Specifications

- Input supply voltage: 1.5 to 3.0 V DC

- Current consumption: <2 microamps (Sleep mode) , 2 micro amps(Active mode)

- Sensing temperature range: -25 deg C to +60 deg C

- Accuracy: 0.25 deg C to 0.5 deg C

- Resolution :0.01 deg C

- Response time: < 1 Second

- Maximum transmit power: +13 dBm

- Wireless range: ~150 m (line of sight)

- Antenna Type: PCB

- Dimension: 90 mm x 52 mm x 18 mm

WSN Mesh network

Snapshot of the sensor’s data visualized in the cloud

Wireless Water Metering Sensor Node

# Product description

- Wireless Water metering flow sensors measure the flow reduces water wastage and usage monitoring

- Reports measured consumption data periodically with configurable interval - Minimum (1 sec) / Maximum (1 day)

- Each message from sensor convey flow data and battery voltage data.

- Powered by 2 x AAA or 2 x AA Batteries or one 1 x Rechargeable LI-Ion (3.7V to 4.2V) battery

- Very low standby mode current consumption of 2 uA allows for long battery life

- Sensor operate in the 865-867 MHz license free band in India.

- Measure & send water flow periodically (interval

>= 1 sec) to external entities via network coordinator/gateway

# Product Applications

- Domestic or Industrial water control applications

- Smart water metering/Billing

- Apartments, commercial buildings and communities can build automated reports to analyze usage of water across the community and reduce consumption

# Product Technical Specifications

- Input supply voltage: 3.0 to 5.0 V DC

- Current consumption: <2 microamps (Sleep mode)

- Accuracy: Class 1 / Class 2

- Maximum transmit power: +13 dBm

- Tx. Data rate: 10Kbp

WSN System Architecture

Snapshot of the sensor’s data visualized in the clou

  - Each node has a unique IEEE assigned 64-bit address

  - Wireless range: Around (line of sight)

  - Antenna Type: PCB antenna with U. FL connector

  - Antenna Gain: +3 dBi

  - Supports 2/3-wire pulse output water flow meters

Wireless Weather station

# Product description

- Dual wireless temperature sensors Bosch & Texas instruments make, measures -40 deg C to +125 deg C.

- It contains ambient light & atmospheric pressure sensors

- Reporting interval can be configured between minimum of once every second to maximum of once every day.

- Each message from sensor data conveys current temperature and battery voltage data.

- Up to 64 such sensor nodes can communicate through one WiSense coordinator node (WSN1120CL)

- Powered by AA or AAA batteries or Li-ion with solar charging circuit.

- Long range external dipole antenna

- The radio device operates in the 865-867 MHz license free band in India.

Product Applications

- Weather monitoring (Rain measurement)

- Environmental monitoring (ambient light, temperature & pressure)

# Product Technical Specifications

- Input supply voltage: 5.0 to 8.5 V DC

- Current consumption: <2 microamps (Sleep mode) , 2 micro amps(Active mode)

- Sensing temperature range: -40 deg C to +125 deg C

- Pressure range: 300 to 1100 hpa

- Accuracy: 1% (temperature)

- Accuracy :0.047 mm(rain guage)

- Resolution: 0.125 deg C

- Response time: < 1 Second

- Maximum transmit power: +13 dBm

- Wireless range: 1.5k.m (line of sight)

- Antenna Type: Dipole antenna

- IP 65 rating

WSN Mesh network

Snapshot of the sensor’s data visualized in the cloud